LCP macOS Build Guide

Toolchain setup and build instructions for macOS

# Hardware Required

- Tag Connect Plug o'nails: TC2030-CTX

- SEGGER J-Link EDU Mini V2

- USB to TTL serial adapter: FT232RL

# Step 1: Install Homebrew

Homebrew is the package manager used to install most of the required tools. Open Terminal (Cmd+Space, type Terminal) and run:

/bin/bash -c "$(curl -fsSL https://raw.githubusercontent.com/Homebrew/install/HEAD/install.sh)"

# Step 2: Install Toolchain

Run the following commands in Terminal:

brew install cmake ninja

brew install --cask gcc-arm-embedded

brew install --cask segger-jlink

Install Visual Studio Code:

- Download from: https://code.visualstudio.com/

- Or via Homebrew:

  - brew install --cask visual-studio-code

# Step 3: Set Up System PATH

Homebrew-installed tools (cmake, ninja, arm-none-eabi-gcc) are added to PATH automatically. J-Link must be added manually.

## Find your J-Link version

Run the following to see what is installed:

ls /Applications/SEGGER/

You should see a folder like JLink_V942. Use that folder name in the next step.

## Add J-Link to PATH

Replace JLink_V942 with your actual version if different:

echo 'export PATH="/Applications/SEGGER/JLink_V942:$PATH"' >> ~/.zshrc

source ~/.zshrc

## Create JLink.exe symlink

The build system looks for JLink.exe (the Windows name). Create a symlink so it resolves correctly on Mac:

sudo ln -s /Applications/SEGGER/JLink_V942/JLinkExe /usr/local/bin/JLink.exe

Note: Replace JLink_V942 with your actual version. You will be prompted for your Mac password.

## Verify all tools are accessible

arm-none-eabi-gcc --version

cmake --version

ninja --version

JLinkExe --version

Each command should print a version number. If any return 'command not found', the PATH is not set correctly.

# Step 4: Clone Required Repositories

## AmbiqSuite SDK

Clone the AmbiqSuite SDK into your project folder (adjust path as needed):

git clone https://github.com/sparkfun/AmbiqSuiteSDK ~/Downloads/LCP/AmbiqSuiteSDK

Note: The folder must be named exactly AmbiqSuiteSDK — CMake looks for this name. If cloned with a different name, rename it with: mv <old-name> AmbiqSuiteSDK

## EDD-LCP_Control Firmware

Clone the Ver1.0 branch of the firmware repository:

git clone --branch Ver1.0 https://github.com/NOAA-PMEL/EDD-LCP_Control.git ~/Downloads/LCP/EDD-LCP_Control

⚠ Do not include angle brackets < > around the URL. The command above is complete as written.

# Step 5: Build the Firmware

## Navigate to the firmware directory

cd ~/Downloads/LCP/EDD-LCP_Control/Firmware/LCP_Control

## Create and enter the build directory

mkdir build

cd build

## Run CMake

cmake .. -DCMAKE_TOOLCHAIN_FILE="../arm-gcc-toolchain.cmake" -G "Ninja"

Expected output includes:

- Using Ambiq Suite SDK Path: .../AmbiqSuiteSDK

- Configuring done / Generating done

Note: A warning about JLink.exe not found is expected and harmless if the symlink from Step 3 was created. CMake will still configure correctly.

## Flash the firmware

With the J-Link connected, board powered on, and Tag Connect seated on the board:

ninja flash_jlink

## Alternative flash (RS232 to TTL)

Only works when there is no error state on the control board (i.e. not the first flash, and the logger has correct pressure profiles):

ninja bootload_asb

# Step 6: Rebuilding After Config Changes

When making changes to config.h, delete the build directory and repeat the build steps:

cd ~/Downloads/LCP/EDD-LCP_Control/Firmware/LCP_Control

rm -rf build

mkdir build && cd build

cmake .. -DCMAKE_TOOLCHAIN_FILE="../arm-gcc-toolchain.cmake" -G "Ninja"

ninja flash_jlink

# Serial Terminal (TeraTerm Equivalent)

TeraTerm is Windows-only. On macOS, use one of the following:

## screen (built-in, no install needed)

First find your device name after plugging in the FT232RL adapter:

ls /dev/tty.usb*

Then connect at 921600 baud for the Quick Start procedure:

screen /dev/tty.usbserial-XXXX 921600

To exit screen: press Ctrl+A, then Ctrl+\, then confirm with 'y'.

Note: For the Firmware Flash procedure (piston board), use baud rate 115200 instead.

## CoolTerm (free GUI app)

- Download from https://freeware.the-meiers.org/

- Set baud rate to 921600

- Set line ending to LF only (Options > Terminal > Enter Key Emulation: LF)

- Enable logging via Connection > Capture to Text File

# Troubleshooting

## CMake error: Ambiq Suite SDK path not found

The AmbiqSuiteSDK folder is missing or misnamed. Verify it exists:

ls ~/Downloads/LCP/

If the folder is named differently (e.g. AmbiqSuiteSDK-main), rename it:

mv ~/Downloads/LCP/AmbiqSuiteSDK-main ~/Downloads/LCP/AmbiqSuiteSDK

## CMake warning: JLink.exe not found

This warning is expected on Mac. As long as the symlink from Step 3 was created correctly, ninja flash_jlink will still work. Verify with:

which JLink.exe

Should return /usr/local/bin/JLink.exe. If not, re-run the symlink command from Step 3.

## ninja flash_jlink: Cannot connect to the probe/programmer

The firmware compiled successfully but J-Link cannot reach the hardware. Check:

- J-Link EDU Mini is plugged into USB

- Board is powered on

- Tag Connect TC2030-CTX is fully seated on the board header

For more detail on the connection failure, run:

JLinkExe -device AMA3B1KK-KBR -if SWD -speed 4000

## git clone: zsh parse error near newline

You included angle brackets around the URL. Remove them — the URL should be pasted directly with no surrounding characters:

git clone --branch Ver1.0 https://github.com/NOAA-PMEL/EDD-LCP_Control.git